MOKO Beacon Advertising Data Format

Contents

1. Generic	3

1.1 Raw Data	3

1.2 Standard Apple iBeacon	4

1.3 Standard Google Eddystone-UID	4

1.4 Standard Google Eddystone-URL	5

1.5 Standard Google Eddystone-TLM (Unencrypted)	6

2. Moko Standard Firmware Adv Format	7

2.1 MkiBeacon Firmware	7

2.1.1 Response packet	7

2.1.2 MkiBeacon - 3-axis Acc sensor data (Response packet)	7

2.2 BeaconX Firmware	8

2.2.1 BeaconX - Device Info	8

2.2.2 BeaconX - iBeacon	8

2.3 BeaconX Pro (BXP) series Firmware	9

2.3.1 BeaconX Pro - Device Info	9

2.3.2 BeaconX Pro - iBeacon (Response packet)	10

2.3.3 BeaconX Pro - 3-axis Acc sensor data	10

2.3.4 BeaconX Pro - T&H sensor data	13

3.Appendix	15

3.1 Tx Power Chart	15

Version record	16

1. Generic

1.1 Raw Data

No format filtering， all BLE advertising data.

1.2 Standard Apple iBeacon

1.3 Standard Google Eddystone-UID

1.4 Standard Google Eddystone-URL

URL Scheme Prefix chart:

| Decimal | Hex Value | Expansion |
| --- | --- | --- |
| 0 | 0x00 | http://www. |
| 1 | 0x01 | https://www. |
| 2 | 0x02 | http:// |
| 3 | 0x03 | https:// |

Eddystone-URL HTTP URL encoding chart:

| Decimal | Hex Value | Expansion |
| --- | --- | --- |
| 0 | 0x00 | .com/ |
| 1 | 0x01 | .org/ |
| 2 | 0x02 | .edu/ |
| 3 | 0x03 | .net/ |
| 4 | 0x04 | .info/ |
| 5 | 0x05 | .biz/ |
| 6 | 0x06 | .gov/ |
| 7 | 0x07 | .com |
| 8 | 0x08 | .org |
| 9 | 0x09 | .edu |
| 10 | 0x0a | .net |
| 11 | 0x0b | .info |
| 12 | 0x0c | .biz |
| 13 | 0x0d | .gov |
| other | 0x0e~0x20; 0x7F~0xFF | Reserved for Future Use |

1.5 Standard Google Eddystone-TLM (Unencrypted)

2. Moko Standard Firmware Adv Format

2.1 MkiBeacon Firmware

2.1.1 Response packet

MkiBeacon ADV packet = Standard Apple iBeacon advertising format + Customized response packet.

The response packet does not contain iBeacon UUID.

It is for the iOS OS to display the iBeacon data.

2.1.2 MkiBeacon - 3-axis Acc sensor data (Response packet)

MkiBeacon - 3-axis Acc sensor data = Standard Apple iBeacon advertising format + Customize response packet.

The response packet does not contain iBeacon UUID.

It is for the iOS OS to display the iBeacon data and 3-axis accelerometer sensor data.

2.2 BeaconX Firmware

2.2.1 BeaconX - Device Info （Always advertise）

2.2.2 BeaconX - iBeacon

It is for the iOS OS to display the iBeacon data.

2.3 BeaconX Pro (BXP) series Firmware

2.3.1 BeaconX Pro - Device Info

2.3.2 BeaconX Pro - iBeacon (Response packet)

BeaconX Pro - iBeacon = Standard Apple iBeacon advertising packet + Customized response packet.

It is for the iOS OS to display the iBeacon data.

2.3.3 BeaconX Pro - 3-axis Acc sensor data

This format applies the old firmware version - BeaconX Pro V3.x.x series. The default 3-axis accelerometer data is 8bit, and the lower 8 bits is 0b0000 0000.

Conversion formula:

If the original data is less than 0x8000:

Result=(RAW>>8) * s;

If the original data is greater than or equal to 0x8000:

Result=((RAW>>8)-0x100) * s;

| Sampling mode | Full-Scale | s |
| --- | --- | --- |
| Low-power mode (8-bit data output) | ±2g | 16 mg/digit |
|  | ±4g | 32 mg/digit |
|  | ±8g | 64 mg/digit |
|  | ±16g | 192 mg/digit |

For example : Device lies in flat desk and Z-axis down to desk, and keep static status. 
Preconditions:

Sampling mode --- Low power mode

Full-Scale ---±2g

Data output : X_RAW =0xFF00; Y_RAW =0xFB00; Z_RAW =0xC000;

Formula:

X_result=((0xFF00>>8)-0x100)*16 mg/digit= (0xFF-0x100)*16 mg/digit=-1*16=-16mg;

Y_result=((0xFD00>>8)-0x100)*16 mg/digit= (0xFD-0x100)*16 mg/digit=-3*16=-48mg;

Z_result=((0xC000>>8)-0x100)*16 mg/digit= (0xC0-0x100)*16 mg/digit=-64*16=-1024mg;

This format applies the new firmware version - BXP，BXP-D and other BXP series firmware version. The default 3-axis accelerometer data is 12bit, and the lower 4 bits is 0b0000.

Conversion formula:

If the original data is less than 0x8000:

Result=(RAW>>4) * s;

If the original data is greater than or equal to 0x8000:

Result=((RAW>>4)-0x1000) * s;

| Sampling mode | Full-Scale | s |
| --- | --- | --- |
| High-resolution mode (12-bit data output) | ±2g | 1 mg/digit |
|  | ±4g | 2 mg/digit |
|  | ±8g | 4 mg/digit |
|  | ±16g | 12 mg/digit |

For example : Device lies in flat desk and Z-axis down to desk, and keep static status. 
Pre-condition 1:

Sampling mode --- High resolution mode

Full-Scale ---±2g

Data output : X_RAW =0xFE40; Y_RAW =0xFE00; Z_RAW =0xC0C0;

Formula:

X_result=((0xFE40>>4)-0x1000)*1 mg/digit= (0x0FE4-0x1000)*1 mg/digit=-28*1=-28mg;

Y_result=((0xFE00>>4)-0x1000)*1 mg/digit= (0x0FE0-0x1000)*1 mg/digit=-32*1=-32mg;

Z_result=((0xC0C0>>4)-0x1000)*1 mg/digit= (0x0C0C-0x1000)*1 mg/digit=-1012*1=-1012mg;

Pre-condition 2:

Sampling mode --- High resolution mode

Full-Scale ---±8g

Data output : X_RAW =0xFF80; Y_RAW =0xFF80; Z_RAW =0xF040;

Formula:

X_result=((0xFF80>>4)-0x1000)*4 mg/digit= (0x0FF8-0x1000)*4 mg/digit=-8*4=-32mg;

Y_result=((0xFF80>>4)-0x1000)*4 mg/digit= (0x0FF8-0x1000)*4 mg/digit=-8*4=-32mg;

Z_result=((0xF040>>4)-0x1000)*1 mg/digit= (0x0F04-0x1000)*4 mg/digit=-252*4=-1008mg;

Sampling data rate chart:

| Hex Value | Data Rate |
| --- | --- |
| 0x00 | 1Hz |
| 0x01 | 10Hz |
| 0x02 | 25Hz |
| 0x03 | 50Hz |
| 0x04 | 100Hz |

User selectable scale chart:

| Hex Value | Scale |
| --- | --- |
| 0x00 | ±2g |
| 0x01 | ±4g |
| 0x02 | ±8g |
| 0x03 | ±16g |

2.3.4 BeaconX Pro - T&H sensor data

This format applies the old firmware version - BeaconX Pro V3.x.x series.

This format applies the new firmware version - BXP series firmware version.

3.Appendix

3.1 Tx Power Chart

| Hex Value | Tx Power |
| --- | --- |
| 0x04 | +4 dBm |
| 0x03 | +3 dBm |
| 0x00 | 0 dBm |
| 0xFC | -4 dBm |
| 0xF8 | -8 dBm |
| 0xF4 | -12 dBm |
| 0xF0 | -16 dBm |
| 0xEC | -20 dBm |
| 0xD8 | -40 dBm |

Version record

| Version | Data | Notes | Contributor |
| --- | --- | --- | --- |
| V1.0 | 2021.07.09 | Initial version | Daniel |
| V1.1 | 2021.12.10 | 1. Modify formula of 3-axis accelerometer data which expressed in BXP app. | Daniel |
| V1.2 | 2021.12.16 | 1. Modify advertisement format of device info advertisement and 3-axis Acc advertisement. | Daniel |